Type 1.B

Type 1.B – Paragraph Form

Product shall be ECTC Type 1.B, which is comprised of natural and/or polymer fibers mechanically interlocked and/or chemically adhered together to form a netless RECP.  Product shall have a C Factor ≤ 0.10 from standardized large‐scale rainfall performance testing, ASTM D6459 or equivalent deemed acceptable by the engineer.  Product unvegetated permissible shear stress rating shall be ≥ 1.0 lbs/ft2 (≥ 48 Pa) according to ASTM D6460 or equivalent deemed acceptable by the engineer.  MD (Machine Direction) tensile strength shall be ≥ 125 lbs/ft (≥ 1.8 kN/m) x TD (Transverse Direction) tensile strength of ≥ 10 lbs/ft (≥ 0.1 kN/m) according to ASTM D6818.  Product shall have a thickness ≥ 0.3 in (≥ 7.6 mm) according to ASTM D6525, ground coverage of ≥ 50% - ≤ 90% according to ASTM D6567, and mass per unit area of ≥ 10.0 oz/yd2 (≥ 339 g/m2) according to ASTM D6475.

Type 1.B – Tabular Form

| ECTC Type | 1.B |
| --- | --- |
| Product Description | Netless Rolled Erosion Control Blankets |
| Material Composition | Natural and/or polymer fibers mechanically interlocked and/or chemically adhered together to form a netless RECP |
| C Factorb | ≤ 0.10 |
| Shear Stressc | ≥ 1.0 lbs/ft2 (≥ 48 Pa) |
| MD Material Tensile Strength (ASTM D6818) | ≥ 125 lbs/ft (≥ 1.8 kN/m) |
| TD Material Tensile Strength (ASTM D6818) | ≥ 10 lbs/ft (≥ 0.1 kN/m) |
| Material Thickness (ASTM D6525) | ≥ 0.30 in (≥ 7.6 mm) |
| Ground Coverage (ASTM D6567) | ≥ 50% - ≤ 90% |
| Mass Per Unit Area (ASTM D6475) | ≥ 10.0 oz/yd2 (≥ 339 g/m2) |

a. C Factor and permissible shear stress for Types 1.A. and 2.A. mulch control nettings must be obtained with netting used in conjunction with pre‐applied mulch material.

b. ASTM D6459 or equivalent deemed acceptable by the engineer.

c. ASTM D6460 or equivalent deemed acceptable by the engineer.